Student name:

Help received:

EE431: Digital Signal Processing

Virginia Military Institute

Laboratory 4: The Discrete Fourier Transform (DTFT)

Problem 1:  DTFT analytically known

Consider the signal x[n] whose DTFT is

a)	Using Matlab, create a neatly-labeled figure using subplot whose upper plot is the magnitude of X(ejω) and the lower plot is the phase angle of X(ejω) for frequencies -4π  ω  4π.  Is the DTFT a periodic function of ω?  If so, what is the period?  Explain the type of symmetry exhibited by each of the two subplots.

< Paste plotting commands here. >

< Insert MATLAB figure here.  Copy from figure window and paste. >

b)	Challenge: Analytically determine x[n] by computing the inverse DTFT using tables.

< Insert response here. >

c)	Now consider a different signal whose DTFT is

and create a figure using subplot whose upper plot is the magnitude of U(ejω) and whose lower plot is the phase angle of U(ejω) for frequencies 0  ω  π.

< Paste plotting commands here. >

< Insert MATLAB figure here.  Copy from figure window and paste. >

Is this primarily a high frequency or a low frequency signal, or something else?

< Insert response here. >

Can you explain the jump in the phase angle?

< Insert response here. >

The jump can be removed with the Matlab command unwrap.  Replot just the phase spectrum with the jump removed.

< Paste plotting commands here. >

< Insert MATLAB figure here.  Copy from figure window and paste. >

Problem 2:  DTFT not analytically known

a)	Consider the signal g[n] = [1  3  5  7  9  11  13  15  17] that begins at n=0 (i.e. g[0]=1, etc.).  Using the method given in the section "DTFT not analytically known" plot the magnitude spectrum and phase spectrum of the sequence over 0 to π.

< Paste plotting commands here. >

< Insert MATLAB figure here.  Copy from figure window and paste. >

Comment on the distribution of signal energy.  Can you explain the jumps in the phase spectrum?

< Insert response here. >

b)	How would you modify your code to plot the phase spectrum in degrees instead of radians?  You need only show the new "plot" single-line command.

< Paste plot command here. >

c)	Challenge: Analytically determine G(ejω).

< Insert response here. >

Now use freqz to generate a magnitude spectrum.

< Paste plotting commands here. >

< Insert MATLAB figure here.  Copy from figure window and paste. >

Compare with the magnitude spectrum generated in part a.

< Insert response here. >

Problem 3:  Time-shift property of the DTFT

a)	Although the programmer received top grades from his professor for clever plotting, he was boned by the TAC for not including comments or axis labels.  What does variable D do?  (You don't need to include any plots for any of the parts of Question 3, except the challenge.)

< Insert response here. >

What should the horizontal axis and vertical axis labels read (what are their units)?

< Insert response here. >

b)	Examine the two systems H1 and H2 by plotting the impulse response of h1[n] and the impulse response of h2[n] (which is just time-delayed h1[n]).  Remember, these are discrete systems, so use stem plots.

< Insert response here. >

c)	Examine the DTFT of H1 and H2 using the above code.  Explain what adding the delay in the time domain did to the magnitude and phase spectrum plots.  Qualitative comments are good, but quantitative comments are better.

< Insert response here. >

d)	Experiment with a shorter and longer-length signal (i.e. invent a different, shorter signal, and compare how its DTFT changes when not delayed vs. when delayed, and then do the same for a different, longer signal you invent).  Does the length of the signal affect how its phase varies with a delay?

< Insert response here. >

e)	Challenge:  If the phase of the DTFT of a real sequence x[n] is a constant that does not change with frequency (e.g. X(ejω) = a constant vs. something like X(ejω) = ω) , what can you say about the sequence x[n]?  What values could the phase be?

< Insert response here. >

Problem 4:  Convolution property of the DTFT

a)	Run the program and comment on the program results.  What does it show happens to the DTFT magnitude and phase when two signals are convolved in the time domain?

< Insert response here. >

b)	Challenge: The phase of both signals look quite smooth, except for a couple of small jags.  You may have noticed similar small jags in the phase spectra of other signals.   These are artifacts, meaning they shouldn't be there, and are caused by some inaccuracy in the way Matlab calculates those particular values.  What causes them?  Why are they difficult to overcome?  Hint: take a look at the magnitude spectrum.

< Insert response here. >

Problem 5: Real-World Applications of the DTFT

a.	Examine the code given above for DTFTsampled.  Why does it only compute the DTFT of a sampled signal at discrete-time frequencies corresponding to continuous-time frequencies of Fsample/2?  That is, why does it not compute the DTFT at higher discrete-time frequencies that correspond to continuous-time frequencies of Fsample or higher?

< Insert response here. >

b.	A 5 minute sample of a volunteer's EKG was taken and posted on the network as ekgdata.mat.  The EKG was sampled at 360Hz.  The algorithm given above is accurate, but too slow to analyze the entire 5min * 60sec/min * 360samples/sec = 108,000 long data vector.  Rather than analyze all the data, just analyze the first 2000 samples (recall you can extract a subvector in Matlab from a large vector by indexing it, like this:  x([1:2000])  ).  It may still take a while to compute.  Include a plot of the first 2000 samples of the EKG in the time-domain and a plot of the DTFT spectrum magnitude (i.e. the energy distribution vs. frequency) of the first 2000 samples of the  EKG.

< Paste plotting commands here for the time-domain EKG plot >

< Insert time-domain Matlab figure here>

< Paste plotting commands here for the DTFT of the EKG plot >

< Insert the EKG DTFT spectrum magnitude figure here>

c.	Most of the EKG energy should be in the under-20Hz region.  Use the zoom-in and zoom-out tools of the Matlab plot window to examine the area from 0 to 20Hz in greater detail.  To do this, select the zoom-in icon (the magnifying glass with the plus) and drag it in the white plot area to select.  Select the same-sized vertical region (otherwise the signal will not be visible) and the 0 to 20Hz horizontal area.  If you select the wrong area you can select the zoom-out tool (the magnifying glass with the minus) and repeatedly click the plot until the original view is restored.  Alternatively, you can zoom-in precisely by selecting the menu options Tools  Edit Plot, and then double-click the horizontal axis.  Do this until you can precisely locate the first frequency peak indicating the fundamental heartbeat; this will probably be somewhere in the 0.8 to 1.5 Hz region.  What is it for the given data?  What is the heart rate in beats per minute?

< Insert response here. >

d.	You identified the fundamental frequency in part c.  Since the EKG of a cardiac cycle does not look exactly like a sine wave, you know from your EE230 studies of the Fourier Series that a first harmonic will be present at a frequency twice the fundamental, a second harmonic and at frequency 3 times the fundamental, and so on.  How many of these harmonics can you identify in the band below 10 Hz?

< Insert response here. >

e.	Zoom back out by selecting the zoom-out key and clicking in the white area a number of times (or faster, by using the menu option Tools  Reset View).  The EKG is a small signal of about 1mV intensity, and can be affected by capacitively-coupled (through the air) 60Hz powerline noise from wires running overhead and in the floor.  Fluorescent lights often add a 120Hz harmonic.  Using the zoom-in or direct plot editing as described in part c, identify whether each of these noise sources are present, and if so what their peak power is compared to the peak power of the fundamental heartbeat.  That is, find the peak in the magnitude DTFT corresponding to the 60Hz noise (if present), find the peak corresponding to the fundamental of the heart beat, and divide the two to find the strength of the 60Hz noise relative to the signal (the inverse of the signal to noise ratio).  It will probably be somewhere in the 1-25% area for the 60Hz noise.

< Amplitude of the fundamental. >

< Amplitude of the 60Hz peak. >

< Ratio. >

f.	Challenge: Heart rate is directly influenced by breathing rate; it speeds up when you breathe in and slows on the exhale.  Using the zoom-in and out capabilities of Matlab, use the EKG to identify the breathing rate of our test subject in breaths-per-minute.   Include a plot showing the data you used to determine the breathing rate.

< Insert MATLAB figure here.  Copy from figure window and paste. >

< Insert answer here. >

The sampling rate is a little coarse here for a precise response; what could be done to get a more accurate reading?

< Insert answer here. >